Versions 5.6, 6.0 and 6.2

How to route specific traffic to one member of the SD-WAN

Description

This article shows an example on how to route traffic from specific source to one member of the SD-WAN.

Scope

FortiGate/FortiOS 5.6.0, FortiOS 6.0.x and FortiOs 6.2.x

Configuration example

In this configuration example we will route traffic coming from “subnet” to port1, only in case of link or interface failure

the traffic from “subnet” will be routed to port2

- Create address object

config firewall address

edit subnet

set subnet 192.168.1.1 255.255.255.0

next

- Configure SD-WAN members

- Configure Performance SLA

- Configure SD-WAN rule on CLI with mode manual including only the preferable member

config system virtual-wan-link

config service

edit 1

set name subnet-to-port1

set mode manual

set member 1

set dst all

set src subnet

end

the config should be as below:

config system virtual-wan-link

set status enable

config members

edit 1

set interface "port1"

set gateway 10.1.1.2

next

edit 2

set interface "port2"

set gateway 20.2.2.2

next

end

config health-check

edit "8.8.8.8"

set server "8.8.8.8"

set members 1 2

next

end

config service

edit 1

set name "subnet-to-port1"

set member 1

set dst "all"

set src "subnet"

next

end

end

- Generate traffic from “subnet” to verify that it is using the correct interface

To verify run a sniffer to capture the traffic coming from “subnet”

4.979750 port1 out 10.1.1.1 -> 8.8.4.4: icmp: echo request

4.981440 port1 in 8.8.4.4 -> 10.1.1.1: icmp: echo reply

4.981444 mgmt out 8.8.4.4 -> 192.168.1.1: icmp: echo reply

5.996049 mgmt in 192.168.1.1 -> 8.8.4.4: icmp: echo request

5.996053 port1 out 10.1.1.1 -> 8.8.4.4: icmp: echo request

5.997690 port1 in 8.8.4.4 -> 10.1.1.1: icmp: echo reply

5.997692 mgmt out 8.8.4.4 -> 192.168.1.1: icmp: echo reply

7.027688 mgmt in 192.168.1.1 -> 8.8.4.4: icmp: echo request

In case the link will fail the SD-WAN rule will be ignored and the traffic will be routed to port2

To verify run a sniffer to capture the traffic coming from “subnet”

3.368375 port2 out 20.2.2.1 -> 8.8.4.4: icmp: echo request

203.370026 port2 in 8.8.4.4 -> 20.2.2.1: icmp: echo reply

203.370029 mgmt out 8.8.4.4 -> 192.168.1.1: icmp: echo reply

204.384004 mgmt in 192.168.1.1 -> 8.8.4.4: icmp: echo request

204.384006 port2 out 20.2.2.1 -> 8.8.4.4: icmp: echo request

204.385619 port2 in 8.8.4.4 -> 20.2.2.1: icmp: echo reply

204.385622 mgmt out 8.8.4.4 -> 192.168.1.1: icmp: echo reply

205.400076 mgmt in 192.168.1.1-> 8.8.4.4: icmp: echo request

Notes:

This configuration example is focusing on how to create the SD-WAN rule and the Performance SLA in order

not to load balance specific traffic.

To configure the SD-Wan service you should follow the information on below doc:

https://docs.fortinet.com/document/fortigate/6.0.0/cookbook/990932/redundant-internet-with-sd-wan