Versions 5.6, 6.0 and 6.2

Technical Tip: How to route traffic from VXLAN to other VLANs or interfaces.

Description

This article describes how to route traffic coming from VXLAN to other VLANs or interfaces

Scope

FortiGate/ FortiOs5.6.x,  FortiOS 6.0.x and FortiOs 6.2.x

Solution

To configure VXLAN you can follow kb: https://kb.fortinet.com/kb/microsites/search.do?cmd=displayKC&docType=kc&externalId=FD46412

(This article describes how to configure native VXLAN without encryption.)

In order to allow Traffic from VXLAN to reach destinations that do not belong to the VXLAN range you should follow the below steps:

- Assign IP address in the switch interface:

The VXLAN configuration requires a switch interface as below:

# config system switch-interface

edit "name"

set vdom "root"

set member "vxlan-int" "port"

next

end

The switch interface will be created:

edit "name"

set vdom "root"

set type switch

set snmp-index 34

next

The switch interface should be further configured:

config system interface

edit "name"

set vdom "root"

set ip <IP from the range of vxlan>

set allowaccess ping

set vlanforward enable

set type switch

end

- Then firewall policies should be created to allow traffic from the switch interface to the interface or the VLAN you want to reach. NAT should be enabled were it is needed.